| MODEM | USB CONN | MAC Label | MAC Loc. | HTML Interface | Light Configuration | Walled Garden | BOOST? | Stand By? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3Com CMX | N | EA= | Back | NONE | Power & Cable Solid (Left two lights) | None | N | N |
| 3Com Home Connect | Y | EA= | Back | 192.168.100.1 | 3Com & Bulls-eye (Bottom two lights) | None | N | N |
| Terayon Terajet | N | MAC | Bottom | NONE | Power & Cable Solid (Top two lights) | Power light Solid, Cable flashes | N | N |
| RCA DCM266 | Y | MAC | Dark Grey | 192.168.100.1 | Power & Cable Solid (1st & 3rd from right) | None | N | N |
| SA Webstar DPC2100 | Y | MAC | Back Or Side | 192.168.100.1 | Power & Cable Solid (1st & 4th from right) | All lights flash simultaneously | Y | N |
| DPX2203 | Y | MAC | Back Or Side | 192.168.100.1 | Power & Cable Solid (1st & 4th from right) | All lights flash simultaneously | Y | N |
| DPC2203C | Y | MAC | Back Or Side | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | All lights flash simultaneously | Y | N |
| Motorola SB3100 | N | HFC MAC ID | Back | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | None | N | Y |
| SB4100 | Y | HFC MAC ID | Back | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | None | N | Y |
| SB4200 | Y | HFC MAC ID | Back | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | None | N | Y |
| SB5100 | Y | HFC MAC ID | Bottom | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | Power light flashes | Y | Y |
| SBV4200 | Y | HFC MAC ID | Back | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | None | N | Y |
| SBV5120 | Y | HFC MAC ID | Bottom | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | Power light flashes | Y | Y |
| SBV5121XM | Y | HFC MAC ID | Bottom | 192.168.100.1 | Power, Send, Receive, Online (Top Four lights) | Power light flashes | Y | Y |

WALLED GARDEN/BRICKED

Six Walled Garden Reasons:

1. Unregistered/non-provisioned (peach)

2. Non-payment (blue)

3. Change of Service (green)

4. Service Disconnect (green)

5. Gated Garden (yellow)

Virus/hacking/spamming

6. Locked Garden (red)

Terms of Service broken 6+ times

For 5 & 6 sub call (516) 803-2770

Six Escalation Reasons:

1. Missing Rates

2. Missing MAC & JSI/Memberservices cannot resolve the issue

3. MAC on incorrect outlet

4. Wrong equipment type assigned to MAC outlet

5. Duplicate Phone Number

6. MAC registered in old Cabledata acct (fix by removing MAC # in old Acct and register in Correct Acct)

Walled Garden/Modem Registration:

1. Check Old Poll (see if bricked)

2. Check Boost2 (why bricked?);

A. if acct balance Check Ledger and send to customer service

B. COS downgrade check rates and outlets, and get supervisor if rates in CD need to change

3. Run JSI whether OV or OOL modem

4. Run Memberservices if JSI doesn’t work only for non-OV modems

JSI PAGE “MODEM REGISTRATION”

Joint Service Installation

Used for provisioning OOL&OV service it will provision a modem just like Memberservices except it will also do OV so it will get a Dial Tone.  Primary tool for TSG install of OV/OOL modems

CABLEDATA FLIP OPTIONS

Hs – Account History:

NPAY = non-payment

T/C = Trouble Call

IBND = Inbound transfer

CHG = Change of Service

BOX = Change made on box

SRO = Special Request Order

RSTR = Restore

DISC = Disconnect

INST = Install

CP = Complete

XO = Cancelled

RS = Rescheduled

OR = Ordered

ND = Not Done

NT = No Truck

Ld – Ledger:

Shows payments and owed funds

Left side is yellow if OVERDUE

Co – Collections:

Adr – billing address

Rt – Rates:

Must say OOL for service

Ot – Outlets:

Lists all devices on users account

Lists IPT phone number

Tc – Trouble Call:

Dl – Dual Accounts

CABLEDATA T/C PROBLEM CODES

100: No Picture/Sound on TV

150: Modem offline/No connectivity/ should                be checked at main line

152: Possible Defected Modem, Check modem

154: Intermittent Signal / Slow Speeds

155: Verified Packet Loss to Modem

160: Self Install unable to work / OOL rescue

180: No OV Dial Tone (Must for OV)

200: Advised of Possible Charge

301: OOL Modem

302: OV Modem

310: BCOOL Modem

400: Repeat Call / 2nd call within 30 days

420: Customer Requested for T/C and/or refused to troubleshoot.

DHCP

Discover – CPU ask for IP address

Offer – DHCP gives IP w/in scope

Request – Asks for offered IP

Acknowledge – DHCP Grants IP Request

OOL RATES

$29.95 Special Rates

$44.95 Regular Rates

$14.95 Boost w/out OV

$9.95   Boost with OV

$46.95 Relocate Modem or Add A/O

$44.95 OOL Additional Service / 2nd modem with internet service as well

OLD POLLING PAGE

Shows status of devices connected to the modem. Includes following info:

MAC address

Modem address

Down / Upstream interface

IfAlias is the Node

OnlineTekAssign = VOIP

Percentage time online

QoS_Profile = Docsis Version.

Receive power

Carrier / Noise Ratio

Transmit_Power

Ethernet Status = Up / Dormant

QoS Service Flow (WG info)

Attached device section provides information for what device is directly attached to the modem (21., 67., 68., or 69.,)

TROUBLESHOOTING STEPS

Three Troubleshooting Layers:

1. Network to Modem

2. Modem to PC

3. PC OS & Software

The Six Questions

1. What type or Make of Modem do you have?

2. What are the lights on the Modem doing?

3. What is the MAC Address on the Modem?

4. Can you poll the Modem via DOCSIS?

5. Is there an attached device & can you ping?

6. What Operating System & Software is used?

NETWORK TO MODEM

NFOS

Three Reasons:

1. Wrong MAC Address

2. NBS +24 Hours

3. Outage or Latency

NBS Troubleshooting:

1. Bounce modem

2. Check Outage Board

3. Verify TV status (2, 3, 4, and 5)

4. House Health Check

5. Bypass Splitters to Mainline

6. Setup TC if necessary or explain Internal Home Wiring and refer to WIC to replace Splitters / Modems / Coaxial Wire

MODEM TO PC

Router / Hub Steps

1. Bounce Router

2. Check DORA log; if granted check connection, if No Dora Log: Bounce Modem then Bounce Router

3. Check Internet

4. Bypass Router

5. Bounce Modem

6. Refer to Router Manufacturer

Ethernet Troubleshooting Steps

1. Bounce Modem

2. Reseat RJ45

3. Swap Ends to the Connections

4. If Router involved (Router T/S Steps)

5. Verify NIC drivers are installed properly

6. Try another RJ45 cable

USB Troubleshooting Steps

1. Remove USB Drivers from Device Manager

A) Network Adapters

B) Other Devices

C) Universal Serial Bus Controllers

2. Remove USB Cable BEFORE closing Device Manager.  Do Not Reboot.

3. Run the Removal Utility

A) Motorola Surfboard = remove.exe or cleanup.exe or cleanout.exe

1) Surfboard Disc – root of the drive

2) Netsurfer Disc -D:\Drivers\Motorola\CG4501

B) Webstar = UNDPXALL.exe

1) Webstar Disc – root of drive

2) Netsurfer Disc – D:\Drivers\ScientificAtlanta\Webstar

4. Reboot PC – Make sure to have Sub leave disc in the PC

A) Verify if Sub has additional USB devices attached, it is possible that printers, scanners, digital cameras, etc… can affect the modem USB driver install. Disco additional devices if needed

5. Reconnect USB wire – either during reboot or after windows finishes loading.

6. Try another USB wire

7. Walk-through New Hardware Wizard

A) Motorola Surfboard = NETMOTCM.inf

1).Surfboard Disc – root of the drive

2) Netsurfer Disc – D:\Drivers\Motorola\Generic

B) Webstar = WEBSTAR2.inf

1) Webstar Disc – root of drive

2) Netsurfer Disc – D:\Drivers\ScientificAtlanta\DPC2100

8. Verify Connectivity Exists

9. Refer to WIC for new USB cable if needed

Network Interface Card (NIC)

1. Check Device Manager

A) Windows Based PC

1) Start – Run – devmgmt.msc

B) Apple Computer

1) MAC OS 10.X

A) Hard Drive – Applications – Utilities – Network Utility

2) MAC OS 9.X

A) Apple – Control Panels – Extensions Manager – Control Panels –

2. NIC status

1)Yellow Exclamation point – NIC needs driver, refer to PC manufacturer

2) Red X – disabled NIC – right click to re-enable

No Attached Device

1. Check Standby

2. Bounce Modem

3. Reboot the computer

4. Reseat the RJ-45 cable

5. Check the NIC Drivers in Dev. Mgr.

6. Try a different RJ-45

7. Try a different computer

8. Offer to hook up via USB

Attached Device with an IP Address

1. Run IPCONFIG Release/Renew

A) If get an IP address continue below

B) If NO IP address, Follow IPCONFIG errors

2. Ping www.google.com

A) If able to ping, Troubleshoot Browsers

B) If unable to ping continue below

3. Ping 69.147.114.210

A) If able to ping = DNS issue

1) Bypass router if necessary

2) Check Network Settings for STATIC

B) If unable to ping = Possible Winsock issue and firewalls

FIREWALL

1. Verify DORA log is granted

2. Ping Attached Device IP address

A) Unable to ping = Firewall

1) Disable Windows and 3rd Party Firewalls

B) Able to Ping = Refer to PC OS & Software

3. Also Disable any Antivirus / AntiSpyware

BLOCKED PORTS

25 = Blocked to ensure that the OOL network is not being used by spammers

80 = Blocked so that customers can’t run a web server

8080 = Blocked so that customers can’t run a web server

135-139 = NetBIOS allows multiple security holes

445 = NetBIOS allows multiple security holes

1043 = Commonly used for Trojan Virus

1080 = Enables Proxy access

3128 = Enables Proxy Access

PC OS & SOFTWARE

1. Verify Standby Status on Modem

2. Verify Network Settings = Make sure everything set to Automatic = IP address and DNS address

A) Windows Based PC’s

1) 9X, ME

A) Start – Settings – Control Panel – Network – TCP/IP->”Network Card” – Properties

2}2000,  XP

A) Start – Control Panel – Network Connections – Local Area Connection(NIC) – Properties – TCP/IP – Properties

3. Release and Renew IP address to PC

4. Bounce Modem and Bounce PC

5. Disable any Firewalls, Antivirus, and AntiSpyware

6. Troubleshoot Browser issue

BROWSER TROUBLESHOOTING

1. Troubleshoot this issue after verifying that you are able to get a DORA log and ping attached device

2. Run INETWIZ

3. Clear Cookies and Temporary Internet Files

4. Restore Defaults

5. Try another Browser if available

Mozilla Firefox, Netscape, etc.

6. Ping www.yahoo.com

A) If able to ping =  sub is online

B) If unable to ping = try next step

7. Ping 69.147.114.210

A) If able to ping = DNS issue

1) Bypass router if necessary

2) Check Network Settings

B) If unable to ping = Possible Winsock issue

8. Rebuild TCP / IP

A) Windows 98, ME,

1) Start – Settings – Control Panel – Network – ADD – Protocol – ADD – Microsoft on left side – TCP/IP on the ride side – then hit ok

B) Windows 2000

1) Start – Control Panel – Network and Dialup Connection – Local Area Connection – Properties – uncheck Internet Protocol (TCP/IP) – OK – recheck Internet Protocol (TCP/IP) – OK

C) Windows XP

1) Start – Run – netsh int ip resetlog.txt

WINSOCK TROUBLESHOOTING

1. 9X and ME

A) Ping www.yahoo.com

1) ERROR = unknown or unresolved host error

B) Ping 69.147.114.210

1) If able to get replies it is possible Winsock error

2) If unable to get replies it is not a Winsock error

C) FTP 167.206.5.41

1) ERROR = socket 10047, socket 0 are Winsock

2) ERROR = 10061 or 10060 are firewall errors

2. XP

A)Release and Renew IP address

1) ERROR = an operation was attempted on                     something that is not a socket

2) Otherwise NOT WINSOCK ISSUE

B) Telnet mail.optonline.net 110

1) ERROR = WSASTARTUP failed to initialize

A) WINSOCK error, Refer to Vendor

2) IF it goes through it is not a Winsock error

3. SOCKET ERRORS can be looked up at:

http://www.sockets.com/err_lst1.htm#DetailedErrorDescriptions

IPCONFIG ERRORS

1.No operation can be performed on local area connection while its media is disconnected OR  other media state media disconnected messages.

A) These messages are referring to a possible issue with the RJ45 or NIC.

1) Reseat and/or swap out Ethernet cable

2) Disable/Re-enable NIC.

3) Bounce Modem/Router/PC.

4) Rebuilding TCP/IP is a possible resolution.

5) Possible issue with Modem (Is the link/PC Activity light on?  Old poll = USB/Ethernet up/down? Has the sub called in multiple times with a no attached device issue?).

2. Did you release & renew the IP and no information was provided by IPCONFIG?....

A) NIC is disabled or not installed.  Check Device Manager.

B) If all else fails in regards to NIC/adapter issues, offer the USB installation.  If sub is already using USB, and reinstalling drivers doesn't seem to work, try a different USB cable or USB port.  Keep in mind that as of 10/2/07, MAC OS’s and Windows Vista is not compatible with our modems’ USB drivers.

C) Is the sub using a wireless USB adapter?  Maybe a HARDWIRED RJ45 to USB adapter?  Not everyone uses internal adapters.  Disconnect and reconnect the USB adapter, and if necessary, bounce the PC and plug it back in when the PC’s fully booted.

3. No adapters bound to TCP/IP are enabled for DHCP

A) Make sure Internet Protocol TCP/IP is      selected (checked) in the Local Area Connection properties and bounce PC if necessary.

B) NIC may be configured for static IP/DNS, enable DHCP.

C) Rebuild TCP/IP

4. An error occurred while renewing interface local area connection: The system cannot find the file specified.

A) Disable Firewall.  Check for connection.

B) Do you see 0.0.0.0?  Verify NIC is installed.

C) Bounce PC.  Check for connection.

D) Rebuild TCP/IP, bounce PC.  Check for connection.

E) Refer out – Windows XP installation may be damaged, DHCP Client service might not be started, or registry may need editing – REFER OUT!

5.An operation was attempted on something that is not a socket

A) If XP SP2, from "Run" enter netsh Winsock reset, click ok, then reboot PC.

B) Socket registry keys are possibly corrupted – REFER OUT.

C) Possible socket issue due to 3rd party software that use Windows sockets and release/renew commands – REFER OUT.

D) Refer out.

5. Unable to contact DHCP server.  Request timed out. (Especially after 169.254.X.X IP)

A) Unseat RJ45 from both pc & modem, bounce both modem & PC, after PC is fully booted, reseat RJ45 modem first & PC second.  Check for connection.

B) Swap out RJ45, Check for connection.

C) Rebuild TCP/IP and reboot PC.  Check for connection

6. The operation failed as no adapter is in the state permissible for this operation OR The following error occurred when renewing adapter Local Area Connection: DHCP Server unreachable.

A) Adapter not set for DHCP - STATIC - enable DHCP for NIC.

B) TCP/IP may not be installed, check Local Area Connection Properties and make sure Internet Protocol TCP/IP is on the list and selected (checked).

- 7. RPC Server is Unavailable

A) Bounce PC.  Check for connection otherwise refer out.

8. An internal error occurred. Unable to query host name.

A) Bounce PC.

B) TCP/IP Parameters are possibly missing from the Windows registry, refer sub to PC Manufacturer or Microsoft.  Rebuilding TCP/IP will not fix this issue.

PLEASE NOTE:  Many people have multiple adapters, such as their NIC + a wireless adapter.  After having the sub renew their IP, or after listing their network information using IPCONFIG, it's not a bad idea to ask if they have MORE THAN ONE IP address listed on the screen.  This will ensure you're troubleshooting their NIC and not their wireless adapter.  It wouldn't be too cool if you assumed the media disconnected message was referring to their NIC, when it was in fact referring to their wireless adapter.

GENERAL EMAIL INFORMATION

POP3 (incoming): mail.optonline.net / Port 110

SMTP (outgoing): mail.optonline.net / Port 25

NNTP (newsgroups): news.optonline.net / Port 119

User ID: 4-15 alphanumeric

Password: 4-8 alphanumeric

1 Primary / 4 Sub-accounts (15 emails with Boost)

Maximum Recipients: 50

Port 25 blocked w/out Boost

File Attachment: 16MB Webmail

25 MB Email Client

BOOST – File attachment  16MB Webmail

100 MB Email Client

Trueswitch.com/cablevision – forwards emails from past Internet Service Providers to Cablevision

Usernames are not recycled

Supported Email Clients:

Outlook Express 4-6

Outlook 2000 – 2007

Netscape 4, 6, 7

Eudora

Incredimail

Entourage

Mac Mail

Supported Newsgroups

Outlook Express 4-6

Netscape 4, 6, 7

Entourage

Two Ways to setup Email:

1. Netsurfer Installation CD / Provisioning

2. Call TSG and use BOOST2

Setting up Sub-accounts

1. Setup through AMS screen

A) Go to Main Page = Opimum.net – Email Tools – Setup Additional Email accounts – Login to Account Management Screen(AMS) with Primary account

1. Select Create Sub-account – select Optimum ID – then setup password and verify password

2. Setup through BOOST2 under Manage Email Tab

BOOST2

Used for 95% of email issues

Bringing up account:

1. Account Number

2. User ID

3. 7-digit phone

4. Last Name / First Name

Account Information Tab:

Shows primary and secondary accounts

Shows MAC and if Walled

CSO – Common Sign On:

For signing onto Optimum Account Center , Optimum Rewards, and Optimum Voice

Reassign Portal ID to make User ID to CSO

Manage Email Tab:

Defaults to reset password tab

Create / Delete / Undelete User ID’s

Convert Primary = Turn Sub-accounts to Primary

View Challenge Questions

Move / Transfer (moves User ID from old account to NEW

ACCOUNT MANAGEMENT SCREEN (AMS)

SPAM Away options (on / off)

Customer Service (send CS questions)

Reset password / create sub-accounts

WEBMAIL.OPTIMUM.NET

Login w/ user ID & password

SPAM goes in bulk folder

Calendar for BOOST customers

Address Book – Add / Import / Export

Webmail Limits (Email deleting from Server)

Inbox / Drafts – 90 Days (without BOOST)

Unlimited (with BOOST)

Bulk  - 7 days

Trash – 1 Day

CABLEDATA

Three windows for three regions:

NY / Westchester / CT

Opt: NYCT_corp7803_OPT_1

Port: 2000

Home Corp: 7803

Long Island

Opt: LI_corp7801_OPT_2

Port: 2000

Home Corp: 7801

New Jersey

Opt: NJ_corp7866_OPT_3

Port: 2000

Home Corp: 7866

Three Account Query Methods:

1. Phone Number (option 10)

2. Corp / Account number (option 11)

3. Address and Zip (option 12)

Account Main Screen:

House – ACTV if active

Cust – Customer active?

T/C – Trouble Call

SRO – Special Request Order

WIP – Work in Progress

If P/ IN we cannot do anything to account  Xfer to Sales

Status Codes:

ACTV – Active

DISC – Disconnect

NEVR – Never Installed

P/ DS – Pending Disconnect

P/ IN – Pending Install

P/ TC – Pending Trouble Call

P/ CH – Pending Change of Service

P/ SR – Pending Special Request Order

Select Options:

3 – Chance of Service

5 – Reschedule T/C or CH

6 – Cancel T/C

7 – Setup T/C
10 – Change House Data

11 – Change Box Data

12 – Payment and Adjustments

[ = Return

‘ = Done Key

SPEEDS

15Mbps Download / 2Mbps Upload / min. 200Kbps

5 email accounts (15 w/ BOOST)

500MB each (1GB w/BOOST each)

BOOST opens Ports 25 & 80

BOOST options for Webhosting  which is handled by 3rd Party Company – Hostway

BOOST Speed Issues are handled by BCOOL

BCOOL Telephone number = 866-575-8000

SPEEDS TROUBLESHOOTING

Upload Speeds

1. If slow upload , check for modem throttle (15000 in old poll QOS Service Flows)

A)  If throttled leave Remedy in Work In Progress, Assigned-To = QOS team, Adv sub of a callback from a speed specialist to give a callback from 24 -72 hours

DO NOT ADVISE of being THROTTLED

2. Disable any Peer 2 Peer clients such as Kazaa, Limewire, Bittorrent, etc.

3. Check Bandwidth

Download Speeds

1. Check for Packet Loss to the modem

A) If any packet loss t/s issue as if NBS

2. Check Signal Levels

A) If signal is weak again treat issue as if NBS

3. If sub has a Router

A) Bypass Router (This is a must) Router fluctuate speeds

B) Bounce Modem

4. FTP Test

A) In command prompt type in ftp 167.206.5.41

B) Type “ftp” for username and password

C) Type “bin”

D) Type “get test16”

1) If results are over 200 kbps, sub is ok

… 2) If results are less than 200 kbps, disable firewall, check P2P, check for excessive bandwidth usage, ping NIC for packet loss, check NIC drivers

SECURE SITES AND SPECIFIC WEBSITE TROUBLESHOOTING

Secure Site Troubleshooting

1. Make sure sub has SSL checked off in Internet Options – Advanced Tab

2. Start – Run – regsvr32 softpub.dll

3. If sub still unable to login to Secure Sites refer to PC vendor or Microsoft to reinstall browser

## Specific Website Troubleshooting

1. Verify Network Settings and make sure DNS is set to automatic.

2. Disable any Firewalls, Bypass routers, etc.

3. Verify if website is valid – try going to website on your machine

4. Ping website in question

A) If able to ping issue is on the browser

B) If unable to ping website – possible Winsock, DNS, and/or firewall

5. Ping IP address to website

6. Tracert website

Trace Route is a tool to see if customer is able to reach their destination and/or website.

- Command Prompt – tracert www.”website”.com

- In a Trace route you must confirm at the end if it says “trace complete”

When trace is complete it means that it reaches its destination and it’s not our service that’s causing the issue.

C) During trace route if you encounter any hops with “ * “s and it says “Request Timed Out” this just means that there are firewalls or servers that are not advising information back you.

D) If it does not say trace complete and you receive any errors – ASK A LEAD for further assistance

EMAIL TROUBLE SHOOTING

1. Verify Internet Connectivity

A) Online – follow next step

B) Offline – troubleshoot no connectivity

2. Check Outage Board for any current or ongoing issues

3. Verify Optimum ID & password is correct

A) OMNI – Email Tab – Type User & PW – click on AMS and see if you can get into AMS screen

1) If able to get into AMS then user & pw is valid

2) If unable to get into AMS then pw is invalid (reset pw is necessary)

4. Verify Server settings

5. Verify Optimum ID has a MAILHOST in Account Information Tab

A) If sub does not have MAILHOST

1) Convert Portal ID

A) Manage Email Tab – Convert Portal ID – Select Optimum ID – Submit

2) If unable to Convert Portal ID

A) Make sure Account is active

B) Delete User ID – Undelete User ID – Reassign Portal ID

6. Telnet mail.optonline.net 110 – for incoming issues

7. Telnet mail.optonline.net 25 – for outgoing issues

A) if sub unable to telnet

1) Disable any firewalls, bypass routers

2) Make sure you are able to ping attached device.

B) If able to telnet

1) Issue may be software related

8. Verify if sub is able to login through Webmail

9. Bounce PC

10. If sub is still having issues sending email check modem filters for Port 25 Block

1) If no filters – firewalls, routers, etc.

2) If filtered – Advise customer if they had sent a numerous amount of emails, such as 100/an hr – 300 emails in 3 hours.

1) Advise sub that modem has SPAM filter on due to amount of emails being sent

2) Advise sub that the only way to take filter off is by having BOOST

3) Bounce modem to resolve issue BUT DO NOT ADVISE SUB HOW TO RESOLVE ISSUE

11. Bounce modem

12. If sub still having issues either sending or receiving email issue is on Email Client Software, refer to software manufacturer

# REMEDY

Call documentation software – must have a ticket for every call.

Important Buttons:

- Open

- New Request

- Ticket History

Query Account:

- Phone Number (7 digits)

- Account # (78xx, 6 digits, 2 digits)

- MAC Address

All customer information including date service started, locality, and other info.

Bottom Half of Screen:

Contains all ticket information

Ticket Type = OOL

Ticket Origin = Phone

Status = Closed

Assigned_to_Group = TSG

Assign_To = My Name unless escalation to QoS or Foreign Language Team

Region = CWN

Call Center = Shelton

Issue Description = 4 word description of problem

Cause Description = what causes the problem

Resolution Description = “SEE WORK LOG”

Work Log = All work you did to resolve issue

Remedy T/C setup NBS:

Issue Code = 02 No Conn-Signal Related

Issue Description = NBS

Cause Description = modem offline / no signal

Cause Code = No Signal

Resolution Code –1 = Bypass Splitter/Still No Signal

Resolution Code –2 = Resolved with Truck Roll

Resolution Code –3 = Advised of Call Ahead Program

Resolution Code –4 = Advised of Possible Charge

Resolution Code –5 = Someone Over 18 Will Be Home

If Refused to Troubleshoot, put that,

Work Log = Time slot, Date, NBS steps